### Bipedal Platform Evaluation Primary Candidate Analysis

###### 1. Prometheus Robotics - Legs Module

- Technical Specifications:  A bipedal legs module designed for modular humanoids (~1.6 m full height) featuring approximately 12 degrees of freedom (DOF).

- 96V Integration Feasibility:   VERY HIGH.  The modular nature of the Prometheus platform is ideal for isolating the mechanical structure from the stock control electronics.

- Strategic Notes:  This is a prime candidate for a "bare-metal" mechanical purchase. By acquiring only the structural assembly, we can design a 96V power distribution and inverter suite without fighting legacy 48V layouts.

- Action Item: Contact Prometheus Robotics to verify current production status. Request a formal quote for a mechanical-only leg module and seek CAD access.

- https://calendar.app.google/onZQzuJPKeERhBqS9

###### 2. XTRON ATOM P1 Pro

- Technical Specifications:  A specialized ~0.8 m bipedal legs research platform with 12 DOF. Notable for its 3-in-1 modular foot design, which is critical for ground-contact sensing and dynamic stability.

- 96V Integration Feasibility:   MEDIUM-HIGH.  Includes integrated BLDC modules and an open hardware interface.

- Strategic Notes:  Feasibility hinges on whether the integrated BLDC modules can be bypassed. The URDF and open SDK facilitate development once the 96V power stage is integrated.

###### 3. XTRON TRON2 (Biped Leg Config)

- Technical Specifications:  A multi-form modular robot standing ~1.1 m in biped mode. Estimated torque is approximately 120 Nm.

- 96V Integration Feasibility:   MEDIUM.  Uses modular integrated BLDC units.

- Strategic Notes:  The high/low-level open APIs are promising, but the level of access to the internal motor winding terminations will dictate 96V viability.

- Strategic Notes:  Integration necessitates a complete bypass of the stock integrated BLDC joint modules. We must ascertain whether Foxtech/LimX will facilitate "bare-metal" procurement through mechanical-only assemblies or provide the detailed electrical interface documentation required for custom driver replacement.

###### 4. Roboto Origin (Atom01 / RoboParty)

- Technical Specifications:  A DIY-centric humanoid (~1.2 m) utilizing DaMiao (DM) 4340P and 10010L actuators. Licensed under GPL-3.0.

- 96V Integration Feasibility:   HIGH.  The architecture features 4 USB-to-CAN interfaces, which are vital for per-joint inverter control.

- Strategic Notes:  Because the BOM is fully open, we can replace the stock 48V driver boards with custom-designed 96V GaN CAN-controlled modules. The open firmware allows for the necessary communication timing adjustments required for higher bus voltages.

###### 5. Asimov Open-Source Humanoid Legs

- Technical Specifications:  Adult-scale (~0.9 m leg length) with 12 DOF. Features a strategic RSU ankle and passive toe design.

- 96V Integration Feasibility:   VERY HIGH.  This is our top pick for CAD-driven R&D.

- Strategic Notes:  Licensed under  CERN-OHL-S-2.0 , this platform is entirely actuator-agnostic. The RSU ankle and passive toe design specifically reduce the total actuator count, directly minimizing the number of expensive 96V GaN inverter modules required for a full build while maintaining validated kinematics.These platforms represent the path of least resistance for hardware modification. By moving away from integrated, proprietary "smart servos," we gain the structural freedom required to implement the high-voltage power architecture necessary for next-generation torque density.

#### 2. Commercial Reference Platforms & Opportunistic Procurement

Commercial OEM platforms serve as vital system-level reference models for benchmarking kinematic performance.. Strategically, we could target "defective or mechanical shells" from these OEMs—specifically Unitree—to acquire professional-grade kinematics at a fraction of the cost, effectively using them as mechanical test rigs for our 96V power systems.

##### Unitree Platform Deep-Dive

Unitree G1 EDU:

- Status:  Leading research reference (~1.27 m) with 43 DOF and 120 Nm peak torque.

- 96V Strategy:  Use the G1 EDU to characterize gait loads and knee performance via the open SDK. This data is critical to inform the design of a separate 96V test rig with matched kinematics. The modular joints allow for a proof-of-concept bypass where a custom GaN unit drives a stock motor.

Unitree R1 EDU:

- Status:  A 1.21 m humanoid with 26 DOF.

- 96V Strategy:  This unit is RobotShop Kingston-accessible , making it the most viable option for fast-track North American procurement. The EDU variant offers extra ports and documented disassembly guides (screw-based access to joint modules).

- Technical Warning: .While these systems are less flexible than open-source projects, their availability and professional builds offer a potential benchmark and known kinematics

#### 3. Specialized Open-Source Leg Assemblies & Actuator-Agnostic Designs

By being "actuator-agnostic," these designs lack the legacy 48V electrical constraints of turnkey systems, allowing us to specify 96V-optimized motors and custom GaN drive stages from the start, however require full build/design from us with unknown kinematics.

##### Platform Evaluations

Berkeley Humanoid Lite Leg Assembly

- Cost:  ~$5,000 (Full BOM).

- DOF:  6 (3 per leg).

- CAD Status:  Fully Open (STEP/STL).

- 96V Compatibility:  VERY HIGH. Features 3D-printed cycloidal housings and a simple CAN bus interface, allowing for seamless custom actuator integration.

AGILOped Leg Assembly (ETH Zurich)

- Cost:  ~$1,500/leg (Mechanical only).

- DOF:  5 joints/leg.

- CAD Status:  Fully Open.

- 96V Compatibility:  VERY HIGH. The 4-bar linkage structure is ideal for distributing the high torque generated by custom 96V GaN actuators.

MIT Mini Cheetah Leg (Adapted)

- Cost:  ~ $800–$ 2,000/leg.

- DOF:  3 DOF/leg.

- CAD Status:  Fully Open; Includes motor design.

- 96V Compatibility:   The Ben Katz FOC controller design is the foundational architecture for this project. Scaling this design to 96V involves specifically redesigning the GaN FET stage and adjusting the gate drive circuitry to handle higher voltage transients.

Open-Source Leg (OSL - U Michigan)

- Cost:  ~$23,000 (NSF-funded).

- DOF:  2 (Knee/Ankle).

- CAD Status:  Fully Open.

- 96V Compatibility:  VERY HIGH. An adult-scale prosthetic design with significant institutional credibility. The Maxon EC motor interface is easily swappable for 96V-rated alternatives.

ASPIR v2

- Cost:  ~ $2,500–$ 5,000 (BOM).

- DOF:  12 in legs.

- CAD Status:  Fully Open.

- 96V Compatibility:  HIGH (Structural only).

- Critique:  The stock version uses low-torque standard servos. This platform should be viewed strictly as a low-cost structural frame; a total electrical gutting is mandatory to support custom 96V modules.

#### 4. Full-Scale Humanoid Research & Industrial Platforms

Large-scale platforms offer human-scale presence but typically introduce significant proprietary blockers. The choice here is between high out-of-the-box capability and the "openness" required for power-stage intervention.

##### Platform Catalog

- K-Scale K-Bot:  A 1.4 m platform with 30 DOF. Founded by ex-Meta/Tesla/Boston Dynamics engineers, this platform offers VERY HIGH feasibility because the PCB designs and Robstride QDD motor specs are open-source. This allows for a total redesign of the power distribution for 96V.

- Berkeley Humanoid Lite (Full):  0.8 m height, extendable via carbon fiber leg links. Actuator modules are researcher-designed, making them perfect for 96V GaN "drop-in" upgrades.

- CCBR Humanoid (Caltech/Columbia/Berkeley/Rutgers):  Specifically designed for modular hardware research. Collaboration with the CCBR team on their hardware library is a high-value strategic move for early 96V integration.

- Fourier GR2:  1.75 m height with 230 Nm peak torque. While the SDK is open, it uses proprietary servos, making inverter swaps MEDIUM feasibility and likely requiring a formal partnership.

- XTRON T170A:  A professional 1.7 m platform with a torque density of 220 Nm/kg. It is single-leg jump capable, serving as a performance benchmark for the power density we aim to achieve with 96V GaN.

- Agility Robotics Digit:  HIGH capability but LOW feasibility. Actuators are proprietary and closed; unsuitable for inverter research without restricted partnership access.

- Apptronik Apollo:  1.73 m height with NASA/UT Austin heritage. Cable-driven architecture makes it unique, but its proprietary nature limits independent electrical modification.

5. Recommendations:

Path A: Rapid Prototyping (Commercial Integration)  Procure the  Prometheus Robotics Legs or  XTRON ATOM P1 Pro . These offer the fastest route to a balanced bipedal base. The objective is to negotiate a "mechanical-only" supply, allowing us to immediately integrate 96V GaN hardware into a proven kinematic frame with integrated GRF sensing (P1 Pro).

Path B: Deep Power Architecture Research (Open Strategy)  Select the  Asimov Open-Source Humanoid Legs . Asimov is the superior choice for ground-up GaN research due to its CERN license, which provides the legal freedom to modify and redistribute the power-train design. It offers the lowest "mechanical tax" for implementing a custom 96V DC-link and modular power bus.

Next Steps:

- Procurement:  Send message to Prometheus and Foxtech for "bare structure" mechanical assemblies.

- CAD Analysis:  Download Asimov and Roboto Origin STEP files to begin DC-link decoupling and gate-driver packaging studies.