Phase 0 Literature Synthesis for 96V GaN Humanoid Joint Drive

Executive Overview

This report distills the papers listed in the Phase 0 literature review into a set of design-relevant insights for a 96 V GaN-based knee/leg inverter for humanoid robots, and augments them with a few closely related works and industrial app notes where they sharpen design targets or fill gaps (e.g., EPC and TI humanoid GaN reference designs). The goal is not a narrative prose lit review, but an outline-ready set of bulletproof claims, numbers, and figures that can be dropped into the thesis structure defined in the Phase0LitReview document.[1][2][^3]

The core themes across the literature are:

- GaN device physics justify a strong efficiency and power-density advantage in the 96 V class over Si MOSFETs, particularly for hard-switched, high-current motor drives.[4][5][^6]

- High-frequency-oriented design (gate loop, layout parasitics, EMI) is the primary practical barrier to realizing GaN’s theoretical benefits in real motor inverters.[7][8]

- Humanoid and robotics-focused works emphasize joint-level constraints: volume inside the joint, thermal management with limited airflow, and the need for high torque bandwidth for whole-body control.[9][10][^11]

- Gate driver requirements at 96 V center on dV/dt immunity, false turn-on avoidance, and minimizing common-source inductance via integration and PCB topology.[12][13][14][15]

- PCB-on-DBC and integrated motor drive approaches show how to close the loop between electrical design and cooling, with double-sided cooling and on-package decoupling to keep loop inductance ~1 nH.[16][17]

- Commercial reference designs (EPC91118, TI humanoid GaN note) give concrete benchmarks for current per phase, switching frequency, footprint, and efficiency that your 96 V knee drive should match or exceed.[2][3][^18]

Your own OmniRetarget-based simulations already show 4.5–6.9 percentage-point efficiency gains and up to ~72% reduction in inverter energy dissipation when moving from a 48 V Si knee inverter to a 96 V GaN design under aggressive gait profiles, directly validating the direction suggested by the literature.[^19]

1. GaN Device Fundamentals at 96 V (Supports Outline Section 1)

1.1 Physics and Figures of Merit

Tutorial and survey papers on GaN power devices converge on the same key physical advantages vs Si:

- Wide bandgap and large critical electric field (∼3.3 MV/cm vs ∼0.3 MV/cm for Si) allow much thinner drift regions at a given blocking voltage, which directly reduces specific on-resistance.[^5]

- High electron saturation velocity supports fast switching with reduced dynamic losses, enabling much higher PWM frequencies than Si without a prohibitive efficiency penalty.[6][5]

- GaN devices typically show an order-of-magnitude lower output and gate capacitances at a given on-resistance than Si MOSFETs, improving the switching figure of merit QgRDS(on).[20][6]

For the 100–150 V region relevant to a 96 V DC bus, lateral GaN-on-Si HEMTs are firmly in their sweet spot: surveys and market reports place GaN as the dominant WBG choice below ~700 V, with cost parity to MOSFETs of similar RDS(on) in many applications. AIP’s physics tutorial notes that devices with VDS,max<200 V are explicitly targeted at dc–dc converters, motor drives, wireless power transfer, LiDAR, and robotics, which aligns directly with a humanoid knee inverter.[21][5][^6]

Key point for outline: In Section 1, you can justify GaN at 96 V by citing the combination of high breakdown field, low specific RDS(on), and low capacitances that together lower conduction and switching losses vs Si MOSFETs at the same voltage rating.

1.2 High-Frequency Design Challenges

Bingyao Sun’s Virginia Tech dissertation on high-frequency GaN converter design emphasizes that the cost of exploiting these advantages is a much stricter layout and gate-drive design: high dV/dt and low device capacitances make oscillation, EMI, and false triggering more likely if parasitics are not tightly controlled. The work demonstrates:[8][7]

- A compact 1 kW, 400 V three-phase inverter switching at 500 kHz, and a 10 kW LLC at 400 kHz, both using 650 V GaN HEMTs, achieving efficiencies up to ∼97.9% and power densities above 100 W/in³.[^7]

- The need for PCB-embedded transformers and careful gate driver power supplies to keep isolation capacitance low and EMI manageable at high frequency.[^7]

Key point for outline: Use this thesis in Section 1 and Section 4 to argue that GaN’s theoretical advantages are real in hardware, provided the design explicitly manages high-frequency-oriented issues.

2. GaN Inverter Topologies for Motor Drives (Supports Outline Section 2)

2.1 Standard Three-Phase Half-Bridge Topology

Both the Virginia Tech work and multiple GaN motor drive application notes assume a conventional three-phase, six-switch inverter for PMSM/BLDC drives, using either discrete half-bridges or integrated half-bridge GaN modules. TI’s TIDA-00909 48 V, 10 A GaN inverter reference design uses three 80 V, 10 A half-bridge GaN modules (LMG5200) with shunt-based phase-current sensing and a 220 µF DC-link capacitor, targeting high-speed motor drives at up to 100 kHz PWM.[18][20][^7]

The EE Power article on high-current GaN motor drives emphasizes that GaN’s lower capacitances allow more devices to be paralleled on a given substrate area to process higher currents, and that motor-drive GaN inverters typically operate up to ~100 kHz, with dV/dt intentionally limited to below ∼10 V/ns to protect motor insulation.[^20]

Key point for outline: In Section 2, you can adopt the standard three-phase half-bridge topology but argue that GaN enables moving to higher PWM frequencies (e.g., 50–100 kHz) while keeping current ripple and switching losses acceptable, especially when paired with appropriately sized phase inductors.

2.2 Dead Time and Current Ripple Tradeoffs

High-speed GaN switching reduces the dead time requirement to tens of nanoseconds vs microseconds for Si, which directly cuts dead-time-induced distortion and improves torque production in FOC drives. However, the same fast edges increase dV/dt-related stress and EMI.[13][20]

PCB-motor plus GaN inverter work from METU and related theses illustrate how very high switching frequencies (around 1 MHz) can virtually eliminate current ripple in low-inductance PCB motors and shrink filter components, achieving around 82–90% efficiency at several hundred watts despite the MHz-class switching. This is at the extreme end of frequency and not directly your operating point, but it demonstrates the efficiency vs ripple tradeoff continuum.[22][23][^24]

Key point for outline: You can position your 96 V inverter in an intermediate range (e.g., 50–100 kHz) where GaN provides clear ripple and bandwidth benefits without entering the MHz regime, while citing the PCB-motor papers as upper-bound demonstrations of what GaN can tolerate.

2.3 Integrated Motor Drive and Inverter Co-Design

The PCB axial-flux PMSM papers link inverter design and motor design tightly: GaN switching allows high PWM frequency, which in turn enables lower phase inductance and more compact PCB stators, but requires careful thermal and mechanical design to keep winding temperatures within limits (e.g., ~59 °C at 270 W with passive cooling and ~82–90% efficiency).[23][24]

Key point for outline: These works support your Section 2 claim that inverter topology and PWM strategy cannot be treated in isolation from the motor’s inductance, thermal limits, and mechanical packaging—particularly in a humanoid knee where the drive is packaged inside the joint.

3. GaN Inverters in Humanoid Robotics (Supports Outline Section 3)

3.1 Humanoid-Specific Constraints and TUM Thesis

The TUM “GaN-based Motor Drive for Humanoid Robots” thesis explicitly frames humanoid servo drives as the most space- and thermally-constrained subsystem in full-body robots, due to the need to fit around 40 PMSMs and their inverters within a human-scale form factor while maintaining smooth motion. The thesis argues that GaN-based motor drives offer faster control, lower switching losses, and more compact designs than Si-based drives for the same power, making them particularly attractive for humanoid limbs.[^9]

Your own simulation study on a Unitree G1-class knee, using OmniRetarget gait profiles, shows that a 96 V GaN inverter can improve system efficiency from roughly 89–93% to ~93–98% across several high-intensity maneuvers, with reductions in total inverter energy dissipation up to ∼71% and peak phase currents reduced by ~28–56%. This aligns qualitatively with the TUM thesis and with industrial claims from EPC and TI that GaN-based humanoid joint drives can achieve higher efficiency and more compact footprints than silicon designs.[3][19][^2]

Key figures/graphs to highlight in this section:

- Your gait-profile plots comparing phase current waveforms and total inverter loss vs time for 48 V vs 96 V; the summary tables showing efficiency and energy dissipated for each profile (e.g., energy dropping from ~12.23 J to ~3.47 J, a ~71.66% reduction, with efficiency gains of ~4.5–6.9 percentage points).[^19]

- EPC91118/EPC91104 marketing graphs showing footprint reduction (~66% smaller than Si-based implementations) and elimination of electrolytic capacitors due to high-frequency operation and MLCC-only DC-link.[11][2]

3.2 Industrial Humanoid GaN Reference Designs

EPC’s EPC91118 reference design is advertised as the first GaN-based motor drive reference specifically optimized for humanoid joints, delivering up to 15 ARMS per phase from a 15–55 V DC input, in a 32 mm diameter circular board that fits inside joint motors. It integrates a monolithic GaN ePower stage, current and voltage sensing, rotor-encoder, microcontroller, RS485, and auxiliary supplies on one compact PCB.[^2]

A TI application note on “Application of GaN FET in Humanoid Robots” underscores similar themes: GaN, especially when combined with integrated drivers, enables higher PWM frequencies, lower losses, and smaller motor drives that better meet humanoid requirements for precision, stability, and endurance.[^3]

The HKUST/GZ project on a high-power-density robot actuator driver based on GaN devices further emphasizes:

- High-efficiency drive hardware tuned for joint-level modules.

- Motor parameter self-identification and self-tuning to maintain performance across the full operating range.

- Compensation of low-speed dead zones to maintain control quality during slow, precise motions.[^10]

Key point for outline: In Section 3, use these industrial and academic works to argue that there is a clear trend toward GaN-based integrated actuator modules for humanoids, and that your 96 V GaN knee inverter positions the robot in line with or ahead of these developments by moving to a higher bus voltage while staying within GaN’s optimal range.

4. Gate Driver Requirements for GaN at 96 V (Supports Outline Section 4)

4.1 Monolithically Integrated GaN Gate Drivers

Basler et al.’s “Monolithically Integrated GaN Gate Drivers – A Design Guide” provides a tutorial-style flow on how to design integrated GaN gate drivers co-packaged with the power HEMT. The key messages:[25][12][^13]

- Integrating the driver with the GaN HEMT minimizes gate loop inductance by eliminating bond wires, package leads, and PCB traces between driver and switch, significantly reducing ringing and overvoltage stress.[^13]

- The paper proposes a step-by-step topology selection flow (single vs split path, mono vs multi-stage, stage composition, rail-to-rail drive) and emphasizes the need to balance switching speed, static power consumption, and input capacitance.[12][25]

- Integrated designs can achieve fast drivers with low losses while maintaining controlled switching waveforms, provided the gate loop requirements (external RG, pull-up/down sizing) are carefully met.[^13]

Implication for your design: At 96 V, you are not yet in the 400–650 V regime of many integrated GaN ICs, but the same gate loop principles apply. If using discrete EPC2305 devices, emulate the monolithic approach at the PCB level: extremely short gate traces, Kelvin source, and tight driver co-location.

4.2 dV/dt Immunity and False Turn-On

A recent GaN HEMT dV/dt immunity study (Perera et al., “Evaluation of GaN HEMT dV/dt Immunity and dV/dt Induced False Turn-On Energy Loss”) specifically analyzes how high dV/dt transitions on one device can capacitively couple into the gate of the complementary device, causing partial or full false turn-on and additional energy loss. The work quantifies:[14][15]

- Thresholds of dV/dt at which false turn-on becomes significant for typical GaN HEMTs.

- The additional energy loss associated with each event.

- The effectiveness of mitigation strategies: negative gate bias, optimized gate resistance, and improved layout that reduces common-source inductance.

Complementary work on discrete, closed-loop dV/dt control gate drivers (IET paper referenced in your list) shows that actively controlling dV/dt via gate driver feedback can keep voltage slopes within safe limits while preserving switching speed, albeit at the cost of increased driver complexity.

Key point for outline: In Section 4, make explicit that at 96 V, your design still experiences high dV/dt edges (tens of V/ns if unconstrained) and must therefore incorporate:

- Tight gate loops and Kelvin-source layout following Basler’s guidance.[^13]

- Possibly slight negative gate bias or at least carefully chosen gate resistors, guided by Perera’s false turn-on thresholds.[15][14]

4.3 Bootstrap vs Isolated Gate Supplies

The Fraunhofer design guide and related GaN gate driver literature discuss bootstrap vs isolated supplies for high- and low-side devices:

- Bootstrap supplies are simple and compact but see high dv/dt stress and must be carefully designed to avoid excessive common-mode transients and ground bounce; isolation capacitance of bootstrap transformers or drivers feeds directly into EMI.[7][13]

- Isolated drivers or power stages provide better common-mode noise performance and scale more cleanly to multi-phase, multi-joint architectures, but add cost and board area.

At 96 V, a bootstrap scheme remains feasible but must be designed with high dV/dt robustness; TI’s GaN inverter reference and EPC’s ePower stages illustrate how integrated drivers and local supplies are commonly implemented in compact motor-drive boards.[18][2]

Key point for outline: Explicitly position your KAN-12 gate driver design within this space: for example, “96 V class, discrete EPC2305 with isolated driver per half-bridge” vs “bootstrap + high-CMTI integrated driver”, and justify with citations.

5. Thermal Management of GaN Inverters (Supports Outline Section 5)

5.1 PCB-on-DBC GaN Modules and Double-Sided Cooling

The PCB-on-DBC GaN power module paper shows how to achieve extremely low loop inductance (~1.03 nH) and low thermal resistance (0.32 K/W) by sandwiching GaN dies and integrated gate drivers between a multilayer PCB and a DBC substrate, enabling double-sided cooling. The module, a 2.7 cm × 1.8 cm half-bridge rated for 400 V/25 A, keeps overshoot to <5% in a 400 V/25 A double-pulse test.[17][16]

This structure is an excellent reference for:

- The kind of hybrid PCB–DBC stack-up that would allow a humanoid joint inverter to shed heat through both the motor housing and attached heatsinks.

- How to place decoupling capacitors and gate drivers in the PCB above the die to minimize loop inductance and thermal impedance simultaneously.[^16]

5.2 Thermal Behavior in PCB Motor + GaN Systems

The PCB axial-flux PMSM work shows that, with appropriate thermal design, an integrated GaN drive can operate at 1 MHz and deliver ~270 W at 0.36 N·m and 5000 rpm with stator winding temperatures around 59 °C, and ~90% efficiency at lower torque. It also proposes methods for measuring PCB stator eddy-current losses, which are relevant when your inverter is directly adjacent to or integrated into the motor structure.[^24]

Key point for outline: In Section 5, pair your own simulated loss distributions (conduction vs switching vs thermal stack) with PCB-on-DBC and PCB-motor results to argue for a specific target thermal resistance and junction temperature budget for the 96 V knee inverter.

5.3 Your Simulation Ties Electrical to Thermal

Your humanoid simulation document already calculates inverter losses and shows how reduced current at 96 V directly lowers resistive heating, enabling the robot to perform high-torque tasks longer without thermal shutdown. It also hints at more advanced control possibilities (e.g., self-sensing, higher torque-loop bandwidth) that are only feasible because GaN devices can switch at 100 kHz with minimal reverse recovery and dead-time losses.[^19]

6. Reference Design Benchmarks and Design Targets (Supports Outline Section 6)

6.1 EPC Humanoid GaN Reference Designs

EPC’s humanoid-focused reference designs and blog posts provide concrete numbers you can treat as benchmarks or design targets:

- EPC91118: 15 ARMS per phase, 15–55 V input, 32 mm diameter inverter board, 55 mm frame, 100 kHz PWM with ~50 ns dead time, MLCC-only DC link, and ~66% footprint reduction vs Si designs.[^2]

- EPC91104 and related designs are positioned for high-precision joints such as wrists and toes, highlighting GaN’s ability to provide high bandwidth torque control in compact formats.[^11]

The blog “Why Humanoid Robots Demand Precision Motor Drives” stresses that humanoids require fast torque response for small joints like toes and wrists to maintain balance and execute dexterous tasks; GaN’s higher speed, efficiency, and compactness are presented as enablers of this requirement.[^11]

6.2 TI Humanoid GaN Application Note

TI’s humanoid GaN application note argues that GaN-based motor drives are likely to become the preferred choice in humanoids due to their ability to combine high power density, low loss, and integrated drivers. It reinforces that high PWM frequencies and integrated gate drivers reduce the size and improve precision, which is important for stable, efficient humanoid robots.[^3]

6.3 Other High-Power-Density GaN Motor Drive Benchmarks

High-frequency GaN power electronics presentations and EMI-focused reviews report that fast-switching GaN HEMTs enable major reductions in magnetic component size and overall converter volume, but require EMI-aware layouts and sometimes modified PWM schemes to maintain efficiency and reduce noise.[26][27][^28]

Key point for outline: Section 6 should tabulate key benchmarks: current per phase, bus voltage, PWM frequency, board diameter/area, efficiency, and thermal resistance from EPC, TI, and PCB-on-DBC modules, then place your 96 V design in that landscape.

7. Suggested Figures, Tables, and How to Use Them in the Thesis

The literature and your own work suggest several high-value visuals to reproduce or adapt (respecting copyright by re-drawing in your style with cited data):

7.1 Device Fundamentals

- A table comparing GaN vs Si vs SiC in terms of bandgap, critical field, saturation velocity, and typical RDS(on) per voltage class, based on tutorial and survey data.[5][6]

- A conceptual figure of specific RDS(on) vs breakdown voltage, inspired by GaN device reviews, showing GaN’s advantage in the 100–200 V region relevant to 96 V buses.[6][5]

7.2 Inverter Topology and Layout

- A block diagram of the three-phase GaN inverter plus PMSM with highlighted gate loops and power loops, citing TI and EE Power layout guidance for gate trace minimization, Kelvin source, and DC-link placement.[29][18][^20]

- A re-drawn version of the PCB-on-DBC GaN module cross-section, showing GaN dies sandwiched between PCB and DBC with double-sided cooling and integrated decoupling capacitors.[^16]

7.3 Humanoid Application Figures

- Plots from your OmniRetarget-based simulations: current waveforms, torque tracking, and time-resolved inverter power loss for one or two “current killer” gait profiles, overlaid for 48 V Si versus 96 V GaN.[^19]

- A bar chart or table summarizing efficiency, total energy dissipated, and peak current for gait profiles 1–5, highlighting the worst-case improvement (~71.66% energy reduction).[^19]

- A schematic of a humanoid leg module with the 96 V GaN inverter indicated in the knee joint, referencing TUM and EPC’s focus on joint-level integration.[9][2]

7.4 Gate Driver and dV/dt Control

- A simplified gate driver topology diagram based on Basler’s design flow (single-path, multi-stage driver) annotated with your chosen gate resistors and supply voltage, to visually tie your KAN-12 design to the monolithic gate-driver literature.[25][12]

- A conceptual plot of false turn-on energy vs dV/dt, adapted from the dV/dt immunity paper, with a shaded “safe operating region” that your layout and driver aim to stay within.[14][15]

7.5 Thermal and Power Density

- A stacked bar chart comparing conduction vs switching loss shares in your 48 V vs 96 V simulations across representative gaits, tying into the thermal management section.[^19]

- A footprint vs current/voltage scatter plot placing EPC91118, TI TIDA-00909, the PCB-on-DBC module, and your target 96 V knee inverter on the same axes, to visually position your design.[18][2][^16]

8. How This Maps onto Your Phase 0 Outline

The outline inside Phase0LitReview already segments the literature into six sections; the synthesis above provides the content to drop under each heading:

- Section 1 – GaN Device Fundamentals: Use GaN physics tutorials and surveys plus the Virginia Tech thesis to justify GaN at 96 V, focusing on bandgap, critical field, RDS(on), and QgRDS(on).[5][6][^7]

- Section 2 – GaN Inverter Topologies: Use Virginia Tech high-frequency inverter work, PCB motor + GaN papers, TI’s 48 V GaN inverter design, and EE Power’s high-current GaN inverter article to discuss half-bridge topology, dead time, bus capacitors, and ripple vs frequency.[22][23][24][20][18][7]

- Section 3 – GaN Inverters in Humanoids: Use the TUM thesis, EPC humanoid reference designs, TI humanoid app note, HKUST’s GaN actuator driver project, and your own simulation results to define the humanoid design space and motivate a 96 V bus.[10][9][2][3][11][19]

- Section 4 – Gate Driver Requirements: Use Basler’s monolithic GaN gate driver guide, the dV/dt immunity and closed-loop dV/dt works, and the Virginia Tech thesis’ gate-drive sections to specify your design constraints for KAN-12.[15][12][25][14][7][13]

- Section 5 – Thermal Management: Use PCB-on-DBC GaN module work, PCB axial-flux PMSM thermal results, and your own loss simulations to bridge electrical design and thermal stack decisions.[23][24][17][16][^19]

- Section 6 – Reference Benchmarks: Use EPC91118/EPC91104 information, TI’s humanoid GaN note, high-frequency GaN presentations, and EMI-focused reviews to define realistic, literature-backed design targets for current, voltage, PWM frequency, footprint, and thermal resistance.[27][28][26][20][2][3][^18]

With this synthesis, the next step is to flesh out each outline section with 2–3 paragraphs of text plus 1–2 key figures/tables, using the bullets and key points above as the backbone for your Phase 0 write-up.

Appendix: Paper-by-paper design reference

1. Future Perspectives on GaN Power Devices and Applications (IEEE invited)

Scope: High-level invited IEEE article on future GaN power device trends and applications.[^30]

Key takeaways:

- Positions GaN as the preferred technology for low- and medium-voltage power conversion (sub-700 V), explicitly including motor drives, dc–dc converters, wireless power, and robotics.[^30]

- Emphasizes monolithic integration (driver + power stage + protection) and advanced packaging as the path to fully exploiting GaN’s speed and reducing parasitics.[^30]

Use in thesis: Section 1 (GaN device fundamentals) for overall context and integration trend.

2. GaN-Based Power Devices in Power Electronics: A Review (Energies 2023 – Musumeci et al.)

Scope: Peer-reviewed survey of GaN device physics, figures of merit, and application range.[^19]

Key takeaways:

- Confirms GaN’s suitability below about 200 V and around 650 V, listing motor drives and compact power converters as key application domains.[^19]

- Reports 2DEG electron mobility around 2000 cm²/Vs, roughly two orders of magnitude higher than SiC MOSFET channels, enabling much lower specific RDS(on) for a given breakdown voltage.[^19]

- States that GaN converters can operate at switching frequencies up to the tens of MHz, enabling large reductions in magnetics and converter volume at the cost of tighter EMI and layout constraints.[^19]

Use in thesis: Section 1 for numerical device property statements and GaN vs Si/SiC comparison; Section 2 for frequency capability context.

3. High-Frequency Oriented Design of GaN Power Converters (Virginia Tech PhD – Sun, 2018)

Scope: Dissertation on designing high-frequency GaN converters and inverters with focus on parasitics, isolation, and EMI.

Key takeaways:

- Demonstrates a 1 kW, 400 V three-phase GaN inverter at 500 kHz and a 10 kW LLC at 400 kHz achieving efficiencies up to about 97.9% and power densities beyond 100 W/in³, contingent on careful gate-loop and layout design.

- Shows that parasitic inductances and capacitances become dominant loss and EMI contributors at hundreds of kilohertz, requiring embedded magnetics and optimized PCB layout to control isolation capacitance and common-mode noise.

Use in thesis: Section 1 (to show what GaN can do in practice) and Section 2/4 (high-frequency design constraints and isolation/gate-drive considerations).

4. GaN-Based Motor Drive for Humanoid Robots (TUM MASc Thesis – Tian, 2025)

Scope: Master’s thesis specifically targeting GaN servo drives for humanoid robots.

Key takeaways:

- Frames humanoid robots as requiring several dozen joint drives packed in a human-scale body, making joint-level inverter volume and thermal management primary constraints.

- Argues that GaN-based motor drives can achieve higher switching frequency, lower loss, and more compact packaging than Si-based drives, enabling higher torque bandwidth and better whole-body control.

Use in thesis: Section 3 to motivate humanoid-specific requirements and to mirror literature review structure; Section 2 for topology and packaging context.

5. Electrothermal Design of a GaN-Based Axially Stator Iron-Mounted Fully Integrated Modular Motor Drive (Ebrahimian et al.)

Scope: Electrothermal design of a high-power GaN IMMD where each stator coil has its own GaN full-bridge.

Key takeaways:

- Application: More-Electric Aircraft IMMD with 18 GaN full-bridge modules driving a three-phase SPM machine.

- Ratings: Vdc ≈ 1 kV, P ≈ 250 kW, coil current ≈ 268 ARMS, rated electrical frequency ≈ 1250 Hz, efficiency target ≥ 98%, maximum junction temperature 125 °C.

- Uses EPC2034C (200 V, 48 A, 8 mΩ, Coss ≈ 641 pF) and finds that eight devices in parallel per switch at 20 kHz meet current and thermal constraints.

- Realizes a 16-layer, 2 oz copper PCB of 2.92 × 2.26 × 0.08 in, corresponding to a board-level power density around 26 kW/in³.

- Extracts HF commutation loop inductance ≈ 0.7 nH and shows turn-off overshoot ≈ 13% on a 166 V DC bus in double-pulse simulations.

Use in thesis: Section 3 (as a strong IMMD analogue) and Section 5 (electrothermal design, target loop inductance, junction temperature, and power density benchmarks).

6. PCB Axial-Flux PMSM + GaN Integrated Drive (Tokgöz et al., METU)

Scope: Theses and papers on optimized PCB axial-flux PMSM with GaN inverter at very high switching frequency.

Key takeaways:

- Demonstrate a PCB stator PMSM driven by a GaN inverter at around 1 MHz, delivering about 270 W at 0.36 N·m and roughly 5000 rpm with overall efficiency in the range of 82–90%, depending on operating point.

- Report stator winding temperatures on the order of 59 °C under passive cooling at these conditions, validating that densely packed PCB windings plus high-frequency GaN switching can be thermally managed with proper design.

- Provide methods for measuring and modelling eddy-current losses in PCB stators, ensuring that the chosen switching frequency and layout do not cause unacceptable additional losses.

Use in thesis: Section 2 (extreme frequency/ripple tradeoff case study) and Section 5 (thermal behaviour of integrated PCB motor + GaN systems).

7. Monolithically Integrated GaN Gate Drivers – A Design Guide (Basler et al., IEEE JEDS 2023)

Scope: Tutorial-style design guide for integrated GaN gate drivers co-packaged with power HEMTs.[^31]

Key takeaways:

- Proposes a topology-selection flow for GaN drivers (single vs split path, stage composition, rail-to-rail output), linking these choices to switching speed, static consumption, and input capacitance.[^31]

- Emphasizes that integrating driver and GaN switch minimizes gate-loop inductance, substantially reducing ringing, overshoot, and false turn-on risk at high dV/dt.[^31]

Use in thesis: Section 4 as the main academic reference for your gate-driver architecture and gate-loop layout rationale.

8. Monolithically Integrated and Galvanically Isolated GaN Power Stage (Basler et al., OJPEL 2025)

Scope: Demonstration of a monolithic GaN-on-SiC power stage with on-chip isolated gate driver using a GHz-range transformer link.[^32]

Key takeaways:

- Implements an on-chip spiral transformer driven by a class-D oscillator at roughly 200–530 MHz to transfer both power and data across an isolation barrier using a single 8 V primary-side supply.[^32]

- Achieves propagation delays of about 28 ns (high-to-low) and 70 ns (low-to-high) and rise/fall times of roughly 3.6/6.1 ns when driving a 100 pF capacitive load representative of a 600 V GaN switch, with PWM operation up to 2 MHz.[^32]

- Notes that isolation capability is currently limited by a ~300 nm SiN backend dielectric, implying modest isolation voltage today but a clear path to >10 kV with thicker dielectrics.[^32]

Use in thesis: Section 4 for discussion of future integration directions and tradeoffs of on-chip isolation versus discrete isolated drivers at 96 V.

9. Evaluation of GaN HEMT dV/dt Immunity and dV/dt Induced False Turn-On (Perera et al.)

Scope: Experimental evaluation of false turn-on in GaN half-bridges under high dV/dt.[^33]

Key takeaways:

- Quantifies dV/dt thresholds beyond which false turn-on begins to occur, by measuring gate and drain waveforms in controlled double-pulse tests.[^33]

- Shows that each false turn-on event adds measurable energy loss and device stress, even when the event is brief and partial.[^33]

- Demonstrates mitigation through negative gate bias, optimized gate resistance, and layouts with minimized common-source inductance.[^33]

Use in thesis: Section 4 to justify your chosen dV/dt limits, gate bias, RG values, and Kelvin source layout.

10. A Review of EMI Research of High Power Density Motor Drive Systems (Actuators / MDPI)

Scope: EMI-focused review of compact high-power-density motor drive systems, including WBG-based drives.[^34]

Key takeaways:

- Identifies fast dV/dt from WBG devices as a major source of common-mode and differential-mode EMI, coupling through parasitic capacitances in cables, windings, and PCB structures.[^34]

- Reviews mitigation techniques such as common-mode chokes, shielded cables, optimized grounding and layout, and spread-spectrum modulation.[^34]

- Emphasizes that increasing switching frequency and power density tightens EMI constraints and may force co-design of filters and converters to keep filter volume acceptable.[^34]

Use in thesis: Section 2 and 4 as the academic anchor for your EMI discussion.

11. PCB-on-DBC GaN Power Module Design With High-Density Integration (IEEE / NREL)

Scope: High-density GaN half-bridge module using PCB-on-DBC packaging and double-sided cooling.[^35]

Key takeaways:

- Realizes a 400 V, 25 A GaN half-bridge module with integrated driver and decoupling, measuring loop inductance around 1.03 nH and thermal resistance about 0.32 K/W through double-sided cooling.[^35]

- Demonstrates that overshoot can be limited to under 5% in 400 V/25 A double-pulse tests when electrical and thermal design are co-optimized.[^35]

- Uses a stack-up with GaN dies between a high-density PCB and a DBC substrate, directly relevant to joint-level integration inside a humanoid leg.[^35]

Use in thesis: Section 5 (thermal stack design) and Section 6 (benchmark loop inductance and thermal resistance targets).

12. Impact of GaN Switching Devices on High-Power-Density Inverters (Ahmad, Aalto)

Scope: Dissertation on GaN-based high-power-density converters and inverters, focusing on magnetics and EMI.[^36]

Key takeaways:

- Shows that reverse conduction losses can dominate total conduction loss in GaN-based HERIC inverters at low power factor, and proposes PWM strategies to mitigate these losses.[^36]

- Analyzes a high-step-up converter with multi-phase coupled inductor, demonstrating that fast GaN switching permits significant magnetic size reduction but requires careful modelling of parasitic capacitances to control EMI.[^36]

Use in thesis: Section 2 for inverter topology and control nuances; Section 5 for power-density and EMI tradeoffs influenced by GaN switching characteristics.

13. 96 V GaN vs 48 V Si Efficiency in Humanoid Robotics (Your Simulation Study)

Scope: Simulated comparison of 48 V Si vs 96 V GaN knee inverters on Unitree-class gaits generated by OmniRetarget.[^37]

Key takeaways:

- Shows system efficiency improvements on the order of 4.5–6.9 percentage points when moving from a 48 V Si baseline to a 96 V GaN inverter across several high-intensity gait profiles.[^37]

- Reports worst-case inverter energy dissipation falling from roughly 12.23 J to 3.47 J for a given gait, a reduction of about 71.66%, and peak current reductions of roughly 28–56% depending on profile.[^37]

- Highlights that GaN’s ability to switch at 100 kHz without reverse recovery, combined with reduced dead time, yields smoother phase currents and enables higher torque-loop bandwidth.

Use in thesis: Section 3 and 5 as your primary quantitative evidence for selecting a 96 V GaN architecture for humanoid knees.

14. EPC Humanoid Joint Reference Designs (EPC91118 / EPC91120)

Scope: GaN-based three-phase BLDC inverter reference designs explicitly optimized for humanoid joints.[^38]

Key takeaways:

- EPC91118 delivers up to about 15 ARMS per phase from a 15–55 V bus on a 32 mm diameter inverter board within a 55 mm frame, using monolithic GaN ePower stages.[^38]

- Operates at around 100 kHz PWM with about 50 ns dead time and uses an MLCC-only DC link, resulting in about 66% PCB area reduction versus comparable Si MOSFET designs.[^38]

- Thermal characterization indicates roughly 7 ARMS per phase possible without heatsink and 15 ARMS when thermally coupled to the motor housing.

Use in thesis: Section 3 and 6 as industrial benchmarks for joint-level current density, PWM frequency, board diameter, and DC-link implementation.

15. Application of GaN FET in Humanoid Robots (Texas Instruments App Note)

Scope: Application note positioning GaN motor drives as a preferred architecture for humanoid robots.

Key takeaways:

- Asserts that GaN-based drives, especially when combined with integrated drivers, provide higher PWM frequency capability, lower loss, and more compact implementations than MOSFET-based alternatives for humanoid joints.[^39]

- Highlights the role of high switching frequency and integrated drivers in achieving precise torque and position control in humanoid arms and legs.[^39]

Use in thesis: Section 3 and 6 as secondary support for industry direction and design targets; keep journal and thesis sources as primary evidence.

References

- Phase0LitReview.docx - ------------------------------------------------------------------------

Reading Order

Work th...

- First GaN-Based Motor Drive Reference Design for ... - First GaN-Based Motor Drive Reference Design for Humanoid Robots Delivers up to 15 ARMS in an Ultra-...

- Application of GaN FET in Humanoid Robots - ... GaN based motor drive likely becomes the preferred design in humanoid robots offering a more eff...

- Plenty of Room at the Bottom and Top: Future Perspectives on GaN Power Devices and Applications

- GaN-based power devices: Physics, reliability, and ... - This tutorial describes the physics, technology, and reliability of GaN-based power devices: in the ...

- Gallium Nitride Power Devices in Power Electronics ... - In this article, the impact of the GaN technology in power electronics applications is described and...

- High-Frequency Oriented Design of Gallium-Nitride (GaN ... - The dissertation is developed to address the key high-frequency-oriented challenges to adopt GaN-bas...

- High-Frequency Oriented Design of Gallium-Nitride (GaN ... - In all, this work focuses on providing effective solutions or guidelines to adopt the 650 V GaN HEMT...

- GaN-based Motor Drive for Humanoid Robots - For the same power output, Gallium Nitride (GaN)-based motor drive systems offer faster control, low...

- Development of a High-Power-Density Robot Actuator Driver Based ... - Development of a High-Power-Density Robot Actuator Driver Based on GaN Devices. The micro servo driv...

- Why Humanoid Robots Demand Precision Motor Drives - Evaluation of the Design of a GaN-Based Motor Drive Optimized for Precision Motion in Humanoid Robot...

- Monolithically Integrated GaN Gate Drivers– A Design Guide - A guide is given for the design of a monolithically integrated GaN gate driver to further advance, p...

- Monolithically Integrated GaN Gate Drivers– A Design Guide - Thus, a guide is given for the design of a monolithically integrated GaN gate driver to further adva...

- Evaluation of GaN HEMT dv/dt Immunity and dv/dt Induced ... - Evaluation of GaN HEMT dv/dt Immunity and dv/dt Induced False Turn-On Energy Loss. Abstract: With th...

- Evaluation of GaN HEMT dv/dt Immunity and dv/dt Induced False ... - Evaluation of GaN HEMT dv/dt Immunity and dv/dt Induced False Turn-On Energy Loss · Nirmana Perera, ...

- PCB-on-DBC GaN Power Module Design With High-Density ... - This article discusses a high-density GaN power module with double-sided cooling, low inductance, on...

- PCB-on-DBC GaN Power Module Design with High- ... - This paper discusses a high-density GaN power module with double-sided cooling, low inductance, on-p...

- 48-V, 10-A, High-Frequency PWM, 3-Phase GaN Inverter Reference Design for High-Speed Motor Drives (Rev. B) - (optional)
VA,VB,
VC, VDC
PWM_AH
PWM_AL
PWM_BH
PWM_BL
PWM_CH
PWM_CL
4
6
C2000Œ�
MCU
/DXQFK3DGŒ�
/PWM...

- Humanoid-Robot-Simulation.pdf - Efficiency Analysis of High-Voltage Efficiency Analysis of High-Voltage Architectures for Humanoid R...

- GaN Transistors Simplify High-Current Motor Drive Inverter Design - Technical Articles - # GaN Transistors Simplify High-Current Motor Drive Inverter Design

Battery-powered industrial vehi...

- Where is GaN Going? Gallium Nitride Market, Applications ... - Discover more about Gallium Nitride (GaN)'s expanding market, its applications, and its future: Find...

- Design and Analysis of a GaN-Based Megahertz Integrated ... - Show/Hide Menu

Hide/Show Apps

Logout

Türkçe

Türkçe

Search

Search

Login

Login

- Design and implementation of an optimized printed circuit ... - Design and implementation of an optimized printed circuit board axial-flux permanent magnet machine ...

- Mechanical and Thermal Design of an Optimized PCB Motor for an ...open.metu.edu.tr › handle - Show/Hide Menu

Hide/Show Apps

Logout

Türkçe

Türkçe

Search

Search

Login

Login

OpenMETU

Open...

- Monolithically Integrated GaN Gate Drivers: A Design Guide - In: IEEE Open Journal of Power Electronics. 2023 ; Vol. 4. pp. 487-497. ... Monolithically Integrate...

- Impact of Gallium Nitride Switching Devices on High- ... - In this work a high voltage gain converter with multi-phase coupled inductor, for electric vehicle (...

- High-frequency Power Electronics using eGaN Devices - In this presentation, I will describe the performance of eGaN FETs in. MHz, KW resonant power conver...

- A Review of EMI Research of High Power Density Motor Drive ... - ... Design of High Current, High Power Density GaN Based Motor Drive for All Electric Aircraft Appli...

- Overview of PCB design strategy for GaN motors - With the advent of

wide bandgap semiconductors, **power devices made from gallium nitride (GaN)...

- Legged-Platform-Evaluation-Tracker.xlsx - BIPED HUMANOID LEGGED PLATFORM EVALUATION --- Platform Product Category Source Institution Est. Cost...

- Technische Universität München

- Track: Poster Session WED-PM - CVPR 2026 - ### Session

Poster Session WED-PM

Abstract:

Chat is not available.

001

Highlight

- Track: Poster Session 4 & Exhibit Hall - ### Poster Session

Poster Session 4 & Exhibit Hall

Abstract:

Chat is not available.

Poster #1

- 3D and 4D World Modeling: A Survey

- [PDF] Driving-signal aware full-body avatars - TIMUR BAGAUTDINOV, Facebook Reality Labs, USA
CHENGLEI WU, Facebook Reality Labs, USA
TOMAS SIMON, F...

- QCBCT-NET for direct measurement of bone mineral density from quantitative cone-beam CT: a human skull phantom study - ## Abstract

The purpose of this study was to directly and quantitatively measure BMD from Cone-beam...

- FlowAct-R1: Towards Interactive Humanoid Video Generation - Overview of FlowAct-R1. It consists of training and inference stages: training integrates converting...

- A Review of Multi-Sensor Fusion in Autonomous Driving - PMC - Sensors (Basel). 2025 Oct 1;25(19):6033. doi: 10.3390/s25196033

- BDD100K: Images 100K Dataset - ## Introduction #

Fisher Yu, Haofeng Chen, Xin Wanget al.